SET UP CHECKLIST

Set up the exterior:

- Record mileage

- Evaluate site’s utilities and slide locations before driving in

- Test water

- Test electric, then flip to “off”

- Test cell phones or two-way radios for spotting purposes, if backing in

- Drive in. Make sure all utilities (water, sewer, electric , cable) are reachable

- Walk around RV to confirm position allows for deployment of slide, awning, steps, etc.

- Check for high obstructions

- Level from side to side using blocks

- Place chocks on wheels

- Remove safety chains

- Remove load leveling bars

- Disconnect RV umbilical cord from tow vehicle

- Disconnect breakaway switch from tow vehicle (make sure stairs are up)

- Unhitch

- Level up – Turn off Level Mate Pro

- Place blocks under stabilizer jacks and tongue

- Extend stabilizer jacks

- Plug in electric (turn on breaker)

- Put out slide

- Hook up water line and turn on

- Hook up sewer line (connect sewer hose, make sure both ends are secure, open gray tank valves, empty black tanks if needed, close black tank valves)

- Connect cable TV

- Turn on all propane bottle valves and check for leaks

- Fill water heater tank by opening hot water faucet valve until water comes out

- Turn on propane water heater if needed

- Turn on water pump if no external water source is available

- Open fresh water tank valve

Set up the interior:

- Turn on A/C or heater and set thermostat

- Remove items that may obstruct the slide and put slide out

- Run water until air is through the line

- Confirm that refrigerator is turned on and running on AC

- Remove items from RV which are kept outside

- Remove fasteners used to secure cabinet doors and other items

- Return all items to their normal locations

- Lock all external RV panels

- Clean counters, surface and floor

- Open fridge carefully the first time!